TERMÉKUTASÍTÁS     			           HU

ALPS PROSTHESIS-SOCK

KCN, KCM, KCW

ALPS prosthetic socks are light and soft, providing cushioning and comfort for an excellent fit. The socks are made with Coolmax fibers to create a moisture management system that wicks sweat away from the body through the fabric where it can evaporate quickly.

It adapts to changes in volume and can be worn with or without a prosthesis.

Color coded to identify thickness:

1 layer: Blue

3 layers: Yellow

5 layers: Green

1/4

Function

The device is designed to be used as part of a lower extremity leg prosthesis.

Primarily used over a prosthetic liner to compensate for changes in residual limb volume; It also provides additional protection and cushioning of the skin and helps reducing friction between the prosthesis and the residual limb.

Instructions for use and care

- Wear clean socks daily•

- Use only chlorine-free bleach•

- Wash inside out•

- Tumble dry - delicate cycle, medium heat•

- Machine washing is possible in warm water with gentle detergent

Warnings

Do not wear the socks on cracked or damaged skin

Do not continue to use if the device shows any holes or signs of deterioration.

Guarantee

30-day satisfaction guarantee

The socks are guaranteed to be free of factory defects

2/4

Choosing the right size

1.Measure the distance between the lower end of the residual limb and the top of the prosthetic socket. The length of the appropriate socks for this size is approx. 5 cm larger.

2.Measure the "bottom range" 5 cm above the bottom end.

3.Use the table below to match the measured data with the appropriate ALPS prosthetic sock size.

| Size of the sock | Measured circumference of the stub | Length of the pod (min. – max.) | Description for the socks | Number of the items |
| --- | --- | --- | --- | --- |
| 08 | 20-23 cm | 13-18 cm | Narrow extra short | KCN-08-(layer)(N/H/HR) |
| 10 | 20-23 cm | 18-23 cm | Narrow short | KCN-10-(layer)(N/H/HR) |
| 14 | 20-23 cm | 23-38 cm | Narrow normal | KCN-14-(layer)(N/H/HR) |
| 18 | 20-23 cm | 38-48 cm | Narrow long | KCN-18-(layer)(N/H/HR) |
| 08 | 23-31 cm | 13-18 cm | Medium extra short | KCM-08-(layer)(N/H/HR) |
| 10 | 23-31 cm | 18-23 cm | Medium short | KCM-10-(layer)(N/H/HR) |
| 14 | 23-31 cm | 23-38 cm | Medium normal | KCM-14-(layer)(N/H/HR) |
| 18 | 23-31 cm | 38-48 cm | Medium long | KCM-18-(layer)(N/H/HR) |
| 08 | 31-36 cm | 13-18 cm | Wide extra short | KCW-08-(layer)(N/H/HR) |
| 10 | 31-36 cm | 18-23 cm | Wide short | KCW-10-(layer)(N/H/HR) |
| 14 | 31-36 cm | 23-38 cm | Wide normal | KCW-14-(layer)(N/H/HR) |
| 18 | 31-36 cm | 38-48 cm | Wide long | KCW-18-(layer)(N/H/HR) |

3/4

List of symbols

Designed for a single user

Disposal

The device and packaging must be disposed of according to the local or national environmental regulations.

Check local and national environmental regulations and dispose of them accordingly.

EC Conformity

This product complies with the requirements of the European Regulation EU2017/745 for medical devices. This product is part of the regulation VIII. according to the classification rules described in its annex, it was classified as a class I asset.

Reporting a serious incident

In an unlikely situation when a serious incident occurs with the device, it must be reported to the manufacturer and the competent national authority.

ALPS South LLC			 CEpartner4U 		      ALPS CZECH ENGINEERING s.r.o.

2895 42nd Ave. N 			Esdoornlaan 13, 		Božkovské náměstí 17/21

St. Petersburg, FL 33714, USA 		3951DB Maarn, 		32600 Plzeň

Tel 1-727-528-8566 			The Netherlands 		Czech Republic

Fax 1-727-528-5426 			www.cepartner4u.com             Tel : +420 377 223 127

info@easyliner.com 							Fax : +420 377 223 010

www.easyliner.com 							info@easyliner.eu

www.easyliner.eu

5297 02811.6K6 A

2023. december

4/4